SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE CODE:  CSA0713                    COURSE NAME: COMPUTER NETWORKS

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 40%

Annexure A

ANALYTICAL PROBLEM SOLVING

Code of Conduct:

I S.MOHIT (192312508) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:Mohit

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |
| Method Selection | 20% | Selects most appropriate method/formula with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |

Annexure A

ANALYTICAL PROBLEM SOLVING

1. Suppose we want to transmit the message 11100011 and protect it from errors using the CRC polynomial X3+1.

i.) Use polynomial long division to determine the message that should be transmitted.

ii) Suppose the leftmost bit of the message is inverted due to noise on the transmission link. What is the result of the receiver's CRC calculation? How does the receiver know that an error has occurred?

Answer:

Cyclic Redundancy Check (CRC) is an error-detection technique used in computer networks to determine whether data has been corrupted during transmission. In CRC, the sender performs modulo-2 polynomial division on the data using a predefined generator polynomial. The remainder obtained from this division is appended to the original message and transmitted to the receiver.

Given

Message:11100011

CRC Polynomial:X3+1

The binary representation of the generator polynomial X3+1 is:1001

Since the generator polynomial has a degree of 3, three zeros are appended to the original message before performing the division.

Original message = 11100011

After appending three zeros:11100011 000

Therefore, the dividend is:11100011000

i) Polynomial Long Division

The augmented message is divided by the generator polynomial using modulo-2 division. In modulo-2 division, subtraction is performed using the XOR operation.

11100011000 ÷ 1001

After completing the modulo-2 polynomial long division, the remainder is:

CRC Remainder = 100

The CRC remainder is then appended to the original message.

Transmitted Message=Original Message+CRC

Original Message = 11100011

CRC              = 100

Therefore:

Transmitted Message = 11100011100

Final Answer

Transmitted message=11100011100

​

ii) Error in the Leftmost Bit

Suppose the leftmost bit of the transmitted message is inverted due to noise on the transmission link.

Correct transmitted message:

11100011100

After the leftmost bit is inverted:

01100011100

The receiver performs CRC division on the received message using the same generator polynomial:

01100011100 ÷ 1001

The receiver obtains a non-zero remainder:

Remainder = 100

Since:100 the receiver knows that an error has occurred.

A correctly transmitted CRC codeword should produce a remainder of:000

Therefore, the non-zero remainder indicates that the received data has been corrupted during transmission.

Conclusion

The CRC method successfully detects the error in the leftmost bit. CRC is primarily an error-detection technique. It identifies that the data is corrupted, although it does not directly correct the corrupted bit.

2. a) A network with bandwidth of 10 Mbps can pass only an average of 12,000 frames per minute with each frame carrying an average of 10,000 bits. What is the throughput of this network?

b) A network has a bandwidth of 3000 Hz (300 to 3300 Hz) assigned for data communications. The signal- to-noise ratio is 3162. Find the channel capacity

Answer:

2(a). NETWORK THROUGHPUT

Throughput is the actual amount of data successfully transmitted through a network during a given period of time. It is usually measured in bits per second.

Given

Number of frames = 12,000 frames per minute

Average size of each frame = 10,000 bits

Network bandwidth = 10 Mbps

Step 1: Calculate Total Bits Transmitted per Minute

Total bits per minute=Number of frames×Bits per frame

=12,000×10,000

=120,000,000 bits/minute

Step 2: Convert Bits per Minute to Bits per Second

There are 60 seconds in one minute.

Throughput=60120,000,000=2,000,000 bits/second

Therefore:

Throughput=2 Mbps

​

Step 3: Calculate Network Utilization

The available bandwidth is 10 Mbps.

Utilization=BandwidthThroughput×100=102×100=20%

Final Answer

The network throughput is:2 Mbps

​

The network is utilizing approximately:

20% of its available 10 Mbps bandwidth.

The remaining capacity may be used for additional network traffic, protocol overhead, or sudden increases in data transmission.

2(b). SHANNON CHANNEL CAPACITY

Shannon's theorem determines the theoretical maximum data rate that can be achieved over a communication channel in the presence of noise.

Given

Bandwidth:

B=3000 Hz

Signal-to-noise ratio:

SNR=3162

The Shannon channel capacity formula is:

C=Blog2(1+SNR)

Substituting the given values:

C=3000log2(1+3162)

C=3000log2(3163)

log2(3163)≈11.627

Therefore:

C=3000×11.627

C≈34,881 bits/second

Converting to kilobits per second:

C≈34.88 kbps

Final Answer Channel Capacity≈34.88 kbps

​

This represents the maximum theoretical data rate of the channel under the given bandwidth and signal-to-noise conditions.

3. A multispecialty hospital uses wireless patient-monitoring devices to continuously transmit patients' heart rate and blood pressure readings to a central monitoring station. Since the data is transmitted over a wireless channel, electromagnetic interference occasionally changes one bit during transmission. To ensure that doctors receive accurate patient information without waiting for retransmission, the hospital implements the Hamming (12,8) error-correcting code.

Answer:

3. HAMMING (12,8) ERROR-CORRECTING CODE

The Hamming (12,8) code is an error-correcting code that contains 12 total bits. Out of these 12 bits:

8 bits are data bits.

4 bits are parity bits.

The parity bits are placed at positions that are powers of two:

1,2,4,8

The arrangement is:

Position	1	2	3	4	5	6	7	8	9	10	11	12

Bit Type	P1	P2	D1	P4	D2	D3	D4	P8	D5	D6	D7	D8

Here:

P1, P2, P4, and P8 are parity bits.

D1 to D8 are data bits.

Parity Checking

P1 checks:

1,3,5,7,9,11

P2 checks:

2,3,6,7,10,11

P4 checks:

4,5,6,7,12

P8 checks:

8,9,10,11,12

When a single bit changes during transmission due to electromagnetic interference, the receiver recalculates the parity bits.

The result of the parity checking process is called the syndrome.

The syndrome identifies the exact position of the corrupted bit.

For example, suppose the calculated syndrome is:

P8 P4 P2 P1 = 0110

Converting binary to decimal:

01102=610

​

Therefore, bit position 6 is corrupted.

The receiver corrects the error by changing the value of bit 6:

0 → 1

or:

1 → 0

depending on the received value.

Conclusion

The Hamming (12,8) code can detect and correct a single-bit error without requiring retransmission.

This is especially useful in wireless patient-monitoring systems. If one bit of a patient's heart-rate or blood-pressure data is changed because of electromagnetic interference, the receiver can identify and correct the error immediately.

Therefore, doctors can continue receiving accurate patient information without waiting for the corrupted data to be retransmitted.

4. a) A company uses the IP address 172.16.10.0/24 and has implemented subnetting using a /27 subnet mask for its internal departments. Using the given subnet mask, apply subnetting techniques to calculate the total number of subnets created and the number of usable hosts per subnet. Determine the subnet to which the IP address 172.16.10.78 belongs. Identify the network address and broadcast address for that subnet. Also, determine whether the IP address 172.16.10.95 falls within the same subnet range based on the given configuration.

b) A network uses the Internet Protocol version 6 address 2001:0db8:0000:0000:0000:ff00:0042:8329. Apply IPv6 addressing rules to compress the given address into its shortest form and then expand the compressed address back to its full notation. Using a prefix length of /64, determine the network prefix of the given address. Also, calculate the total number of possible host addresses available within this network based on the given prefix length.

Answer:

4(a). IPv4 SUBNETTING

Subnetting is the process of dividing a large network into smaller logical networks. It improves network organization, address management, performance, and security.

Given

Original network:

172.16.10.0/24

New subnet prefix:/27

Step 1: Calculate Borrowed Bits

27−24=3

Therefore, 3 bits are borrowed from the host portion.

Step 2: Calculate Number of Subnets

2

3

=8

Therefore:

8 subnets

​

Step 3: Calculate Number of Host Bits

IPv4 addresses contain 32 bits.

32−27=5

Therefore, there are 5 host bits.

Step 4: Calculate Total Addresses per Subnet

2

5

=32

Each subnet contains 32 total addresses.

Step 5: Calculate Usable Host Addresses

Two addresses are reserved:

Network address

Broadcast address

Therefore:

32−2=30

Final Subnet Results

8 subnets

​

30 usable hosts per subnet

​

The /27 subnet mask is:

255.255.255.224

The block size is:

256−224=32

Subnet Table

Subnet	Network Address	Usable Host Range	Broadcast Address

1	172.16.10.0	172.16.10.1 – 172.16.10.30	172.16.10.31

2	172.16.10.32	172.16.10.33 – 172.16.10.62	172.16.10.63

3	172.16.10.64	172.16.10.65 – 172.16.10.94	172.16.10.95

4	172.16.10.96	172.16.10.97 – 172.16.10.126	172.16.10.127

5	172.16.10.128	172.16.10.129 – 172.16.10.158	172.16.10.159

6	172.16.10.160	172.16.10.161 – 172.16.10.190	172.16.10.191

7	172.16.10.192	172.16.10.193 – 172.16.10.222	172.16.10.223

8	172.16.10.224	172.16.10.225 – 172.16.10.254	172.16.10.255

IP Address: 172.16.10.78

The last octet is 78.

The third subnet ranges from:

172.16.10.64 to 172.16.10.95

Therefore:

Subnet: 172.16.10.64/27

Network address: 172.16.10.64

Usable host range: 172.16.10.65 to 172.16.10.94

Broadcast address: 172.16.10.95

IP Address: 172.16.10.95

The IP address 172.16.10.95 is the broadcast address of the subnet:

172.16.10.64/27

Therefore, it belongs to the same subnet range as 172.16.10.78, but it cannot be assigned to a normal host device.

Final Answer

172.16.10.78 belongs to 172.16.10.64/27

​

172.16.10.95 is the broadcast address

​

4(b). IPv6 ADDRESSING

IPv6 uses 128-bit addresses. An IPv6 address is written as eight groups of four hexadecimal digits.

Given IPv6 Address

2001:0db8:0000:0000:0000:ff00:0042:8329

Step 1: Remove Leading Zeros

Leading zeros in each group can be removed.

2001:db8:0:0:0:ff00:42:8329

Step 2: Compress Consecutive Zero Groups

The three consecutive zero groups can be replaced by:

::

Therefore, the shortest form is:

2001:db8::ff00:42:8329

​

Step 3: Expand the Compressed Address

The compressed address:

2001:db8::ff00:42:8329

can be expanded to:

2001:0db8:0000:0000:0000:ff00:0042:8329

Step 4: Determine the /64 Network Prefix

An IPv6 address has 128 bits.

For a /64 prefix:

128−64=64

Therefore, the first 64 bits represent the network portion.

The network prefix is:

2001:db8::/64

​

Step 5: Calculate Number of Host Addresses

The remaining 64 bits are available for host addressing.

264

Therefore:

264=18,446,744,073,709,551,616

​

Final Answer

Shortest IPv6 address: 2001:db8::ff00:42:8329

Expanded address: 2001:0db8:0000:0000:0000:ff00:0042:8329

Network prefix: 2001:db8::/64

Number of possible addresses: 18,446,744,073,709,551,616

5. A university is conducting an online semester examination for 8,000 students simultaneously. At exactly 10:00 AM, the examination server begins transmitting encrypted question papers to all students. The university server is connected through a 10 Gbps high-speed network, while students access the examination portal using different internet connections such as fiber broadband (100 Mbps), home Wi-Fi (20 Mbps), and mobile networks (5–10 Mbps). During the first few minutes of the examination, many students with slower internet connections report incomplete downloads, repeated retransmissions, delayed acknowledgments, and temporary freezing of the examination portal. Network administrators observe that the server continues transmitting data at a high rate even though several student devices are unable to receive and process packets at the same speed. This results in receiver buffer overflow, unnecessary retransmissions, increased network traffic, and delays in delivering the question papers. Analyze the above scenario and identify the primary networking problem. Explain how the mismatch between the sender's transmission speed and the receiver's processing capability affects communication.

Answer:

FLOW CONTROL AND RECEIVER BUFFER OVERFLOW

The primary networking problem in this scenario is a flow-control problem caused by a mismatch between the transmission speed of the sender and the processing capability of the receiver.

The university server has a high-speed 10 Gbps network connection. However, the students access the examination portal through different types of networks:

Fiber broadband: 100 Mbps

Home Wi-Fi: 20 Mbps

Mobile networks: 5–10 Mbps

Therefore, the university server may transmit data much faster than some students can receive and process it.

Effect of the Speed Mismatch

The communication process occurs as follows:

The examination server begins transmitting data at a high rate.

Some students have slower network connections.

Data arrives at the receiving device faster than it can be processed.

Packets begin to accumulate in the receiver's buffer.

The buffer eventually becomes full.

New incoming packets may be dropped.

Acknowledgements may be delayed or lost.

The sender may assume that packets have been lost.

The sender retransmits the packets.

Unnecessary retransmissions increase network traffic.

Students experience incomplete downloads and delays.

The examination portal may temporarily freeze.

The main condition can be represented as:

Sender transmission rate>Receiver processing rate

When this condition continues, the receiver buffer may overflow.

Receiver Buffer Overflow

A receiver buffer is temporary memory used to store incoming data before the receiving device processes it.

For example, if:

Incoming data rate = 100 Mbps

but: Receiver processing rate = 20 Mbps

then data arrives faster than it can be processed.

The excess data accumulates in the buffer.

If the buffer becomes full, further packets may be discarded.

This results in:

Packet loss

Delayed acknowledgements

Retransmissions

Increased network traffic

Reduced performance

Incomplete downloads

Temporary freezing of the examination portal

Role of Flow Control

Flow control prevents a fast sender from overwhelming a slow receiver.

In TCP communication, the receiver informs the sender about the amount of data it can currently accept. This information is communicated through the receive window.

The sender then adjusts its transmission rate.

A sliding-window protocol can also be used to control the amount of unacknowledged data sent at one time.

The sender should not continuously transmit data at its maximum possible speed if the receiver cannot process the data at the same rate.

Instead, the sender should adapt its transmission rate according to:

Receiver buffer capacity

Network bandwidth

Available receive window

Network congestion

Processing capability of the receiving device

Conclusion

The main problem in the university examination scenario is the mismatch between the high transmission speed of the server and the lower receiving capability of individual students.

This mismatch causes receiver buffer overflow, packet loss, delayed acknowledgements, unnecessary retransmissions, and delays in delivering examination question papers.

Proper flow-control mechanisms, such as TCP receive-window control and sliding-window protocols, can regulate the amount of data sent to the receiver.

Therefore:

Flow control prevents a fast sender from overwhelming a slow receiver.

​

By controlling the transmission rate, the network can reduce packet loss and retransmissions while improving the reliability and performance of the examination system.

OVERALL CONCLUSION

The five problems demonstrate important concepts in computer networking.

CRC provides a mechanism for detecting errors in transmitted data. Throughput calculations help determine the actual performance of a network, while Shannon's theorem provides the theoretical maximum capacity of a noisy communication channel.

The Hamming (12,8) code provides error correction by detecting and correcting a single-bit error. IPv4 subnetting allows a large network to be divided into smaller subnetworks for efficient address management and network organization. IPv6 addressing provides a very large address space and supports efficient address compression.

Finally, flow control ensures that a fast sender does not overwhelm a slower receiver. This is particularly important in high-traffic systems such as online examinations, where thousands of users may access a server simultaneously.

Together, these concepts are essential for designing reliable, efficient, scalable, and high-performance computer networks.

Summary Table:

Question	Concept	Final Result

1	CRC	Transmitted message = 11100011100; error detected by a non-zero remainder

2(a)	Throughput	2 Mbps; network utilization = 20%

2(b)	Shannon Capacity	Approximately 34.88 kbps

3	Hamming (12,8)	Detects and corrects a single-bit error

4(a)	IPv4 Subnetting	8 subnets; 30 usable hosts per subnet

4(b)	IPv6	2001:db8::ff00:42:8329; /64 prefix; 264 possible addresses

5	Flow Control	Sender is faster than receiver, causing buffer overflow and packet loss