SIMATS ENGINEERING

ASSESSMENT TOOL 2

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA0713

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

Annexure A

Scenario-Based

Code of Conduct:

I S.Mohit(192312508)(Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:Mohit

Annexure A

Scenario-Based

1. A newly established Smart Campus is deploying a secure network

infrastructure for its academic and administrative departments. The network

administrator has been allocated the IP address block 192.168.100.0/24. Based

on the expected number of devices, the following subnet masks have already

been assigned:

School of Computing → /25

School of Electronics → /26

School of Mechanical Engineering → /27

As the network engineer, design the subnet allocation by performing the

following tasks:

1. Determine the network address and broadcast address for each

department.

2. Identify the valid host IP address range for each subnet.

3. Calculate the number of usable host addresses available in each subnet.

4. Determine the remaining unused IP address range within the given

network block.

Given

The Smart Campus has been assigned the IP network 192.168.100.0/24. Based on the number of devices in each department, the following subnet masks are allocated:

- School of Computing → /25

- School of Electronics → /26

- School of Mechanical Engineering → /27

The objective is to determine the network address, broadcast address, valid host range, usable host addresses, and the remaining unused IP address range.

Step 1: School of Computing (/25)

Subnet Mask

255.255.255.128

Number of IP Addresses

Formula:

Total IP Addresses = 2^(32−25)

= 2⁷

= 128

Usable Host Addresses = 128 − 2 = 126

Network Address

192.168.100.0

Broadcast Address

The last address in this subnet is

192.168.100.127

Valid Host Range

First Host = 192.168.100.1

Last Host = 192.168.100.126

Step 2: School of Electronics (/26)

Subnet Mask

255.255.255.192

Number of IP Addresses

Total IPs = 2^(32−26)

= 64

Usable Hosts = 64 − 2

= 62

Network Address

192.168.100.128

Broadcast Address

192.168.100.191

Valid Host Range

First Host = 192.168.100.129

Last Host = 192.168.100.190

Step 3: School of Mechanical Engineering (/27)

Subnet Mask

255.255.255.224

Number of IP Addresses

Total IPs = 2^(32−27)

= 32

Usable Hosts = 32 − 2

= 30

Network Address

192.168.100.192

Broadcast Address

192.168.100.223

Valid Host Range

First Host = 192.168.100.193

Last Host = 192.168.100.222

Step 4: Remaining Unused Address Range

The allocated subnets occupy addresses from

192.168.100.0 to 192.168.100.223

Therefore, the remaining unused addresses are

192.168.100.224 – 192.168.100.255

This unused block contains 32 IP addresses.

Final Answer

| Department | Network Address | Broadcast Address | Valid Host Range | Usable Hosts |
| --- | --- | --- | --- | --- |
| School of Computing | 192.168.100.0 | 192.168.100.127 | 192.168.100.1 – 192.168.100.126 | 126 |
| School of Electronics | 192.168.100.128 | 192.168.100.191 | 192.168.100.129 – 192.168.100.190 | 62 |
| School of Mechanical Engineering | 192.168.100.192 | 192.168.100.223 | 192.168.100.193 – 192.168.100.222 | 30 |

Remaining Unused Range:

192.168.100.224 – 192.168.100.255

2. A multinational software company is expanding its headquarters and has been

assigned the private IP block 10.10.0.0/16. Different departments require

different subnet sizes based on the number of employees. The network

administrator has allocated the following subnet masks:

Software Development → /24

Quality Assurance → /25

Human Resources → /26

Executive Management → /27

As the network administrator, perform the following tasks:

1. Determine the network address and broadcast address for each

department.

2. Identify the valid range of host IP addresses.

3. Calculate the number of usable hosts in each subnet.

4. Identify the unused IP address range remaining within the allocated IP

block.

Given

The company has been allocated the network 10.10.0.0/16.

The departments are assigned the following subnet masks:

- Software Development → /24

- Quality Assurance → /25

- Human Resources → /26

- Executive Management → /27

The task is to determine the subnet details for each department.

Software Development (/24)

Subnet Mask:

255.255.255.0

Total IP Addresses

2^(32−24)

= 256

Usable Hosts

256 − 2

= 254

Network Address

10.10.0.0

Broadcast Address

10.10.0.255

Host Range

10.10.0.1 – 10.10.0.254

Quality Assurance (/25)

Subnet Mask

255.255.255.128

Total IPs

128

Usable Hosts

126

Network Address

10.10.1.0

Broadcast Address

10.10.1.127

Host Range

10.10.1.1 – 10.10.1.126

Human Resources (/26)

Subnet Mask

255.255.255.192

Total IPs

64

Usable Hosts

62

Network Address

10.10.1.128

Broadcast Address

10.10.1.191

Host Range

10.10.1.129 – 10.10.1.190

Executive Management (/27)

Subnet Mask

255.255.255.224

Total IPs

32

Usable Hosts

30

Network Address

10.10.1.192

Broadcast Address

10.10.1.223

Host Range

10.10.1.193 – 10.10.1.222

Unused Address Range

Allocated Range

10.10.0.0 – 10.10.1.223

Remaining Range

10.10.1.224 – 10.10.255.255

Final Answer

| Department | Network | Broadcast | Host Range | Usable Hosts |
| --- | --- | --- | --- | --- |
| Software Development | 10.10.0.0 | 10.10.0.255 | 10.10.0.1 – 10.10.0.254 | 254 |
| Quality Assurance | 10.10.1.0 | 10.10.1.127 | 10.10.1.1 – 10.10.1.126 | 126 |
| Human Resources | 10.10.1.128 | 10.10.1.191 | 10.10.1.129 – 10.10.1.190 | 62 |
| Executive Management | 10.10.1.192 | 10.10.1.223 | 10.10.1.193 – 10.10.1.222 | 30 |

Remaining Unused Range:

10.10.1.224 – 10.10.255.255

3.A smart hospital is migrating its network infrastructure from IPv4 to IPv6 to

support thousands of connected medical devices and IoT sensors. The hospital

has been assigned the IPv6 network:

2001:0db8:abcd:0000:0000:0000:0000:0000/64

As the network engineer responsible for the migration, perform the following:

1. Convert the given IPv6 address into its compressed notation.

2. Expand the compressed address back into its full hexadecimal

representation.

3. Identify the network prefix using the /64 prefix length.

4. Calculate the total number of host addresses supported within the

subnet.

5. the total number of possible host addresses available within this network.

Given

A smart hospital is upgrading its network infrastructure from IPv4 to IPv6 to support a large number of connected medical devices, IoT sensors, and smart healthcare equipment.

The assigned IPv6 network address is:

2001:0db8:abcd:0000:0000:0000:0000:0000/64

The tasks are:

- Convert the given IPv6 address into its compressed notation.

- Expand the compressed address back into its full hexadecimal representation.

- Identify the network prefix using the /64 prefix length.

- Calculate the total number of host addresses supported within the subnet.

- Determine the total number of possible host addresses available within the network.

1. Convert the Given IPv6 Address into Compressed Notation

Given IPv6 Address

2001:0db8:abcd:0000:0000:0000:0000:0000/64

Compression Rules

- Leading zeros in each hexadecimal block can be removed.

- One or more consecutive groups of 0000 can be replaced with ::.

- The double colon (::) can be used only once in an IPv6 address.

Compressed IPv6 Address

2001:db8:abcd::/64

2. Expand the Compressed Address Back to Full Hexadecimal Representation

The compressed address is:

2001:db8:abcd::/64

To expand it:

- Add leading zeros where necessary.

- Replace :: with enough 0000 blocks to make a total of 8 blocks.

Expanded IPv6 Address

2001:0db8:abcd:0000:0000:0000:0000:0000

This is the complete 128-bit IPv6 address.

3. Identify the Network Prefix Using the /64 Prefix Length

The prefix length /64 means:

- The first 64 bits represent the network portion.

- The last 64 bits represent the host (interface identifier) portion.

Network Prefix

2001:db8:abcd:0000::/64

Host Portion

The remaining 64 bits are available for assigning addresses to hosts and devices.

4. Calculate the Total Number of Host Addresses Supported Within the Subnet

Formula

Host Bits = 128 − 64

= 64 bits

Total Number of Host Addresses

= 2⁶⁴

= 18,446,744,073,709,551,616

Thus, the subnet can support:

18,446,744,073,709,551,616 host addresses.

5. Determine the Total Number of Possible Host Addresses Available Within the Network

Since IPv6 uses 64 bits for the host portion,

Total Possible Host Addresses

= 2⁶⁴

= 18,446,744,073,709,551,616

Unlike IPv4, IPv6 does not reserve separate network and broadcast addresses in the same way. Therefore, all addresses within the subnet can be assigned according to IPv6 addressing rules.

Final Answer

| Task | Answer |
| --- | --- |
| Given IPv6 Address | 2001:0db8:abcd:0000:0000:0000:0000:0000/64 |
| Compressed IPv6 Address | 2001:db8:abcd::/64 |
| Expanded IPv6 Address | 2001:0db8:abcd:0000:0000:0000:0000:0000 |
| Network Prefix (/64) | 2001:db8:abcd:0000::/64 |
| Host Portion | Last 64 bits |
| Total Host Addresses | 2⁶⁴ = 18,446,744,073,709,551,616 |
| Total Possible Host Addresses | 18,446,744,073,709,551,616 |

4. An Internet Service Provider (ISP) connects multiple customer networks

through a core router. The router maintains the following routing table entries:

192.168.1.0/24

192.168.2.0/24

192.168.3.0/24

A packet arrives at the router with the destination IP address:

192.168.2.45

As the routing engineer, determine:

1. The destination subnet to which the packet belongs.

2. The matching routing table entry.

3. The next-hop network/interface used to forward the packet.

4. The default route that should be used if the destination does not match

any routing table entry.

Given

An Internet Service Provider (ISP) connects multiple customer networks through a core router. The router contains the following routing table entries:

- 192.168.1.0/24

- 192.168.2.0/24

- 192.168.3.0/24

A packet arrives at the router with the destination IP address:

192.168.2.45

The task is to determine the destination subnet, matching routing table entry, next-hop interface, and the default route if no match exists.

1. Determine the Destination Subnet

The destination IP address is:

192.168.2.45

The subnet mask is /24, which means the first 24 bits (first three octets) represent the network portion.

Network Address:

192.168.2.0/24

Therefore, the destination IP address 192.168.2.45 belongs to the subnet:

192.168.2.0/24

2. Identify the Matching Routing Table Entry

The router checks each routing table entry.

Available entries:

- 192.168.1.0/24 ❌

- 192.168.2.0/24 ✅

- 192.168.3.0/24 ❌

The destination IP matches:

192.168.2.0/24

Therefore, the matching routing table entry is:

192.168.2.0/24

3. Determine the Next-Hop Network/Interface

Once the destination subnet is identified, the router forwards the packet through the interface connected to that subnet.

Since the destination belongs to 192.168.2.0/24, the packet is forwarded through the router interface connected to the 192.168.2.0/24 network.

Note: The exact interface name (such as GigabitEthernet0/1) or next-hop IP address is not provided in the question. Therefore, it cannot be specified.

4. Default Route

If the router does not find any matching entry in its routing table, it forwards the packet using the default route.

The default route is written as:

0.0.0.0/0

This route sends unknown destination packets to the default gateway or upstream router.

Final Answer

| Task | Answer |
| --- | --- |
| Destination IP Address | 192.168.2.45 |
| Destination Subnet | 192.168.2.0/24 |
| Matching Routing Entry | 192.168.2.0/24 |
| Next-Hop Network/Interface | Interface connected to the 192.168.2.0/24 network |
| Default Route | 0.0.0.0/0 |

5. A company establishes two office departments connected through a Layer-3

router. The allocated subnet information is as follows:

Administration LAN → 192.168.10.0/26

Finance LAN → 192.168.10.64/26

The router interfaces are configured with:

Administration Gateway → 192.168.10.1

Finance Gateway → 192.168.10.65

As the network administrator, perform the following tasks:

1. Determine the network address and broadcast address for each LAN.

2. Identify the valid host IP address range.

3. Suggest suitable IP addresses for three computers in each LAN.

4. Identify the default gateway that hosts in each LAN should use for

communication.

Given

A company has two departments connected through a Layer-3 router.

Administration LAN

Network Address:

192.168.10.0/26

Gateway:

192.168.10.1

Finance LAN

Network Address:

192.168.10.64/26

Gateway:

192.168.10.65

The tasks are:

- Determine the network address and broadcast address.

- Identify the valid host IP address range.

- Suggest suitable IP addresses for three computers in each LAN.

- Identify the default gateway for each LAN.

1. Administration LAN

Network Address

192.168.10.0

Subnet Mask

255.255.255.192 (/26)

Total IP Addresses

Formula:

2^(32−26)

= 2⁶

= 64 IP addresses

Usable Hosts

64 − 2

= 62

Broadcast Address

The last IP address in this subnet is:

192.168.10.63

Valid Host Range

First Host:

192.168.10.1

Last Host:

192.168.10.62

2. Finance LAN

Network Address

192.168.10.64

Subnet Mask

255.255.255.192 (/26)

Total IP Addresses

64

Usable Hosts

62

Broadcast Address

192.168.10.127

Valid Host Range

First Host

192.168.10.65

Last Host

192.168.10.126

3. Suggested IP Addresses for Computers

Administration LAN

| Computer | IP Address |
| --- | --- |
| Computer 1 | 192.168.10.2 |
| Computer 2 | 192.168.10.3 |
| Computer 3 | 192.168.10.4 |

Finance LAN

| Computer | IP Address |
| --- | --- |
| Computer 1 | 192.168.10.66 |
| Computer 2 | 192.168.10.67 |
| Computer 3 | 192.168.10.68 |

4. Default Gateway

The default gateway is the router interface used by hosts to communicate with other networks.

Administration LAN

Default Gateway:

192.168.10.1

Finance LAN

Default Gateway:

192.168.10.65

Final Answer

| LAN | Network Address | Broadcast Address | Valid Host Range | Default Gateway |
| --- | --- | --- | --- | --- |
| Administration LAN | 192.168.10.0 | 192.168.10.63 | 192.168.10.1 – 192.168.10.62 | 192.168.10.1 |
| Finance LAN | 192.168.10.64 | 192.168.10.127 | 192.168.10.65 – 192.168.10.126 | 192.168.10.65 |

RUBRICS FOR CALCULATION

| Scenario Based Assessment |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Criteria | Weightage (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| Understanding of Scenario | 20% | Demonstrates clear and complete understanding of the scenario and context | Shows good understanding with minor gaps | Partial understanding; misses key aspects | Misinterprets or fails to understand the scenario |
| Identification of Key Issues | 20% | Accurately identifies all relevant issues and factors involved | Identifies most key issues with minor omissions | Identifies limited issues; some irrelevant points | Unable to identify key issues |
| Application of Concepts | 25% | Applies appropriate concepts effectively to address the scenario | Applies concepts with minor errors | Limited or partially correct application | Unable to apply relevant concepts |
| Decision Making | 20% | Makes well-reasoned, logical decisions with strong rationale | Makes reasonable decisions with some justification | Makes weak decisions with limited justification | No clear decisions made or rationale provided |
| Clarity of Response | 15% | Response is clear, structured, and logically presented | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganized and difficult to understand |